P 3 Redistribution Between RIP and OSPF

Background:

Two online booksellers, Example.com and Example.net, have merged and now need a short-term solution to inter-domain routing. Since these companies provide client services to Internet users, it is essential to have minimal downtime during the transition.

Example.com is a small firm running RIP, while Example.net has a somewhat larger network running OSPF. The diagram identifies R2 as the router that will bridge the two networks. Because it is imperative that the two

booksellers continuously deliver Internet services, you should bridge these two routing domains without interfering with each router’s path through its own routing domain to the Internet.

The CIO determines that it is preferable to keep the two protocol domains shown in the diagram during the transition period, because the network engineers on each side need to understand the other’s network before deploying a long-term solution. Redistribution will be a short-term solution.

Objectives

• Review configuration and verification of RIP and OSPF.

• Configure passive interfaces in both RIP and OSPF.

• Filter routing updates using distribute lists.

• Redistribute static routes into RIP.

• Redistribute RIP routes into OSPF.

• Redistribute OSPF routes into RIP.

• Originate a default route into OSPF.

Step 1: Configure loopbacks and assign addresses.

R1(config)# interface Loopback0

R1(config-if)# ip address 172.16.1.1 255.255.255.0

R1(config-if)# interface Loopback48

R1(config-if)# ip address 192.168.48.1 255.255.255.0

R1(config-if)# interface Loopback49

R1(config-if)# ip address 192.168.49.1 255.255.255.0

R1(config-if)# interface Loopback50

R1(config-if)# ip address 192.168.50.1 255.255.255.0

R1(config-if)# interface Loopback51

R1(config-if)# ip address 192.168.51.1 255.255.255.0

R1(config-if)# interface Loopback70

R1(config-if)# ip address 192.168.70.1 255.255.255.0

R1(config-if)# interface Serial0/0/0

R1(config-if)# ip address 172.16.12.1 255.255.255.0

R1(config-if)# clock rate 64000

R1(config-if)# bandwidth 64

R1(config-if)# no shutdown

R2(config)# interface Loopback0

R2(config-if)# ip address 172.16.2.1 255.255.255.0

R2(config-if)# interface Serial0/0/0

R2(config-if)# ip address 172.16.12.2 255.255.255.0

R2(config-if)# bandwidth 64

R2(config-if)# no shutdown

R2(config-if)# interface Serial0/0/1

R2(config-if)# ip address 172.16.23.2 255.255.255.0

R2(config-if)# clock rate 64000

R2(config-if)# bandwidth 64

R2(config-if)# no shutdown

R3(config)# interface Loopback0

R3(config-if)# ip address 172.16.3.1 255.255.255.0

R3(config-if)# interface Loopback20

R3(config-if)# ip address 192.168.20.1 255.255.255.0

R3(config-if)# interface Loopback25

R3(config-if)# ip address 192.168.25.1 255.255.255.0

R3(config-if)# interface Loopback30

R3(config-if)# ip address 192.168.30.1 255.255.255.0

R3(config-if)# interface Loopback35

R3(config-if)# ip address 192.168.35.1 255.255.255.0

R3(config-if)# interface Loopback40

R3(config-if)# ip address 192.168.40.1 255.255.255.0

R3(config-if)# interface Serial0/0/1

R3(config-if)# ip address 172.16.23.3 255.255.255.0

R3(config-if)# bandwidth 64

R3(config-if)# no shutdown

Step 2: Configure RIPv2.

R1(config)# router rip

R1(config-router)# version 2

R1(config-router)# no auto-summary

R1(config-router)# network 172.16.0.0

R1(config-router)# network 192.168.48.0

R1(config-router)# network 192.168.49.0

R1(config-router)# network 192.168.50.0

R1(config-router)# network 192.168.51.0

R1(config-router)# network 192.168.70.0

R2(config)# router rip

R2(config-router)# version 2

R2(config-router)# no auto-summary

R2(config-router)# network 172.16.0.0

R1# show ip route rip

R2# show ip route rip

Step 3: Configure passive interfaces in RIP.

R1# show ip route rip

R2(config)# router rip

R2(config-router)# passive-interface serial 0/0/1

R2# show ip protocols

R1(config)# router rip

R1(config-router)# passive-interface loopback 0

R1(config-router)# passive-interface loopback 48

R1(config-router)# passive-interface loopback 49

R1(config-router)# passive-interface loopback 50

R1(config-router)# passive-interface loopback 51

R1(config-router)# passive-interface loopback 70

R2(config)# router rip

R2(config-router)# passive-interface loopback 0

Step 4: Summarize a supernet with RIP.

R2# show ip route rip

R1(config)# ip route 192.168.48.0 255.255.252.0 null0

R1(config)# router rip

R1(config-router)# redistribute static

R1# show ip route

R2# show ip route

Step 5: Suppress routes using prefix lists.

R1(config)# ip prefix-list RIP-OUT permit 192.168.48.0/22

R1(config)# ip prefix-list RIP-OUT deny 192.168.48.0/22 le 24

R1(config)# ip prefix-list RIP-OUT permit 0.0.0.0/0 le 32

R1(config)# router rip

R1(config-router)# distribute-list prefix RIP-OUT out serial0/0/0

R2# show ip route rip

R2# show ip rip database

Step 6: Configure OSPF.

R2(config)# router ospf 1

R2(config-router)# network 172.16.23.0 0.0.0.255 area 0

R3(config)# router ospf 1

R3(config-router)# network 172.16.0.0 0.0.255.255 area 0

R3(config-router)# network 192.168.0.0 0.0.255.255 area 0

R3(config)# interface Loopback0

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback20

R3(config-if)# ip ospf network point-to-point

R3(config-if)# nterface Loopback25

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback30

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback35

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback40

R3(config-if)# ip ospf network point-to-point

R3# show ip ospf neighbor

R2# show ip route ospf

R3# show ip route ospf

Step 7: Configure passive interfaces in OSPF.

R3(config)# router ospf 1

R3(config-router)# passive-interface default

R3(config-router)# no passive-interface serial 0/0/1

R3# how ip protocols

Step 8: Allow one-way redistribution.

R2(config)# router rip

R2(config-router)# redistribute ospf 1 metric 4

R2# show ip protocols

R1# show ip route rip

R1# ping 192.168.30.1

R1# traceroute 192.168.30.1

R2(config)# router ospf 1

R2(config-router)# default-information originate always

Step 9: Redistribute between two routing protocols.

R2(config)# router ospf 1

R2(config-router)# no default-information originate always

R2(config-router)# redistribute rip

R3# show ip route ospf

R2(config)# router ospf 1

R2(config-router)# redistribute rip subnets

R3# show ip route ospf