P 6 Configuring OSPF for IPv6

Background

In this lab, you configure static IPv6 addresses and EUI-64 IPv6 addresses on interfaces. You then configure

OSPFv3 to route between the IPv6 networks.

Objectives

• Configure a static IPv6 address on an interface.

• Change the default link-local address on an interface.

• Configure an EUI-64 IPv6 address on an interface.

• Enable IPv6 routing and CEF.

• Configure and verify single-area OSPFv3 operation

Step 2: Configuring the hostname and loopback interfaces.

Router(config)# hostname R1

R1(config)# interface loopback0

R1(config-if)# ip address 10.1.1.1 255.255.255.0 \

R1(config-if)# ipv6 address FEC0::1:1/112

Router(config)# hostname R2

R2(config)# interface loopback0

R2(config-if)# ip address 10.1.2.1 255.255.255.0

R2(config-if)# ipv6 address FEC0::2:1/112

Router(config)# hostname R3

R3(config)# interface loopback0

R3(config-if)# ip address 10.1.3.1 255.255.255.0

R3(config-if)# ipv6 address FEC0::3:1/112

Step 3: Configure static IPv6 addresses.

R1(config)# interface serial0/0/0

R1(config-if)# ipv6 address FEC0::12:1/112

R1(config-if)# clockrate 64000

R1(config-if)# bandwidth 64

R1(config-if)# no shutdown

R1(config-if)# interface serial0/0/1

R1(config-if)# ipv6 address FEC0::13:1/112

R1(config-if)# bandwidth 64

R1(config-if)# no shutdown

R2(config)# interface serial0/0/0

R2(config-if)# ipv6 address FEC0::12:2/112

R2(config-if)# bandwidth 64

R2(config-if)# no shutdown

R3(config)# interface serial0/0/0

R3(config-if)# ipv6 address FEC0::13:3/112

R3(config-if)# clockrate 64000

R3(config-if)# bandwidth 64

R3(config-if)# no shutdown

R1# ping FEC0::12:2

R1# ping FEC0::13:3

R2# ping FEC0::12:1

R3# ping FEC0::13:1

Step 4: Change the link-local address on an interface.

R1# show ipv6 interface serial 0/0/0

R2# show ipv6 interface serial 0/0/0

R1(config)# interface serial0/0/0

R1(config-if)# ipv6 address FE80::1 link-local

R2(config)# interface serial0/0/0

R2(config-if)# ipv6 address FE80::2 link-local

R1# ping FE80::2

R2# ping FE80::1

R1# show ipv6 interface serial 0/0/0

R2# show ipv6 interface serial 0/0/0

Step 5: Configure EUI-64 addresses.

R2(config)# interface fastEthernet 0/0

R2(config-if)# ipv6 address FEC0:23::/64 eui-64

R2(config-if)# no shutdown

R3(config)# interface fastEthernet 0/0

R3(config-if)# ipv6 address FEC0:23::/64 eui-64

R3(config-if)# no shutdown

R2# show ipv6 interface fastEthernet 0/0

R2# show ipv6 interface brief

R2# ping FEC0:23::218:B9FF:FECD:BEF0

R3# ping FEC0:23::218:B9FF:FE92:28D8

Step 6: Enable IPv6 routing and CEF.

R1(config)# ipv6 unicast-routing

R1(config)# ipv6 cef

R2(config)# ipv6 unicast-routing

R2(config)# ipv6 cef

R3(config)# ipv6 unicast-routing

R3(config)# ipv6 cef

7: Configure OSPFv3.

R1(config)# interface loopback0

R1(config-if)# ipv6 ospf 1 area 0

R1(config-if)# interface serial0/0/0

R1(config-if)# ipv6 ospf 1 area 0

R1(config-if)# interface serial0/0/1

R1(config-if)# ipv6 ospf 1 area 0

R2(config)# interface loopback0

R2(config-if)# ipv6 ospf 1 area 0

R2(config-if)# interface serial0/0/0

R2(config-if)# ipv6 ospf 1 area 0

R2(config-if)# interface fastEthernet 0/0

R2(config-if)# ipv6 ospf 1 area 0

R3(config)# interface loopback0

R3(config-if)# ipv6 ospf 1 area 0

R3(config-if)# interface serial0/0/0

R3(config-if)# ipv6 ospf 1 area 0

R3(config-if)# interface fastEthernet 0/0

R3(config-if)# ipv6 ospf 1 area 0

R1# show ipv6 ospf neighbor

R2# show ipv6 ospf neighbor

R3# show ipv6 ospf neighbor

R1# show ipv6 route

R2# show ipv6 route

R3# show ipv6 route