P 5 Configuring BGP with Default Routing

Background

The International Travel Agency (ITA) relies extensively on the Internet for sales. For this reason, the ITA has

decided to create a multihomed ISP connectivity solution and contracted with two ISPs for Internet

connectivity with fault tolerance. Because the ITA is connecting to two different service providers, you must

configure BGP, which runs between the ITA boundary router and the two ISP routers.

Objectives

• Configure BGP to exchange routing information with two ISPs.

Step 1: Configure the hostname and interface addresses.

Router R1 (hostname ISP1) hostname ISP1

interface Lo0

description ISP1 Internet Network

ip address 10.1.1.1 255.255.255.0

interface Serial0/0/0

description ISP1 -> ITA

ip address 10.0.0.1 255.255.255.252

clock rate 128000

no shutdown

end

Router R2 (hostname ITA)

hostname ITA

interface Lo0

description Core router network link 1

ip address 192.168.0.1 255.255.255.0 !

interface Lo1

description Core router network link 2

ip address 192.168.1.1 255.255.255.0

interface Serial0/0/0

description ITA -> ISP1

ip address 10.0.0.2 255.255.255.252

no shutdown

interface Serial0/0/1

description ITA -> ISP2

ip address 172.16.0.2 255.255.255.252

clock rate 128000

no shutdown

end

Router R3 (hostname ISP2)

hostname ISP2

interface Lo0

description ISP2 Internet Network

ip address 172.16.1.1 255.255.255.0

interface Serial0/0/1

description ISP2 -> ITA

ip address 172.16.0.1 255.255.255.252

no shutdown

end

Step 2: Configure BGP on the ISP routers.

ISP1(config)# router bgp 200

ISP1(config-router)# neighbor 10.0.0.2 remote-as 100

ISP1(config-router)# network 10.1.1.0 mask 255.255.255.0

ISP2(config)# router bgp 300

ISP2(config-router)# neighbor 172.16.0.2 remote-as 100

ISP2(config-router)# network 172.16.1.0 mask 255.255.255.0

Step 3: Configure BGP on the ITA boundary router.

ITA(config)# router bgp 100

ITA(config-router)# neighbor 10.0.0.1 remote-as 200

ITA(config-router)# neighbor 172.16.0.1 remote-as 300

ITA(config-router)# network 192.168.0.0

ITA(config-router)# network 192.168.1.0

ITA# show ip route

Step 4: Verify BGP on the routers.

ITA# show ip bgp

ISP1# show ip bgp

ITA# show ip bgp neighbors

Step 5: Configure route filters.

ISP2# show ip rout

ITA(config)# access-list 1 permit 192.168.0.0 0.0.1.255

ITA# clear ip bgp *

ISP2# show ip route

ISP1# show ip route

Step 6: Configure primary and backup routes using floating static routes.

ITA# show ip route

ITA(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.1 210

ITA(config)# ip route 0.0.0.0 0.0.0.0 172.16.0.1 220

ITA# show ip route

ISP1# config t

ISP1(config)# interface loopback 100

ISP1(config-if)# ip address 192.168.100.1 255.255.255.0

ITA# show ip route

ITA# ping

ITA# ping 192.168.100.1 source 192.168.1.1

Step 7: Configure primary and backup routes using a default network and a static route.

ITA(config)# no ip route 0.0.0.0 0.0.0.0 10.0.0.1 210

ITA(config)# no ip route 0.0.0.0 0.0.0.0 172.16.0.1 220

ISP1(config)# router bgp 200

ISP1(config-router)# network 192.168.100.0

ISP1(config-router)# end

ITA# show ip route

ITA(config)# ip default-network 192.168.100.0

ITA# show ip route

ITA(config)# ip route 0.0.0.0 0.0.0.0 172.16.0.1 220

ITA# show ip route

ITA# clear ip bgp 10.0.0.1

ITA# show ip route