P 7 Configuring EtherChannel

Background

Four switches have just been installed. The distribution layer switches are Catalyst 3560 switches, and the

access layer switches are Catalyst 2960 switches. There are redundant uplinks between the access layer and

distribution layer. Usually, only one of these links could be used; otherwise, a bridging loop might occur.

However, using only one link utilizes only half of the available bandwidth. EtherChannel allows up to eight

redundant links to be bundled together into one logical link.

Required Resources

• 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or

comparable)

• 2 switches (Cisco 3560 with the Cisco IOS Release 12.2(46)SE C3560- ADVIPSERVICESK9-M

image or comparable)

• Ethernet and console cables

Step 1: Prepare the switches for the lab and Configure basic switch parameters.

DLS1# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

DLS1(config)# interface range fastEthernet 0/7 - 12

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

Step 3: Configure an EtherChannel with Cisco PAgP.

ALS1# show interfaces trunk

ALS1(config)# interface range fastEthernet 0/11 - 12

ALS1(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS1(config)# interface port-channel 1

ALS1(config-if)# switchport mode trunk

ALS1# show etherchannel summary

ALS2(config)# interface range fastEthernet 0/11 - 12

ALS2(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS2(config)# interface port-channel 1

ALS2(config-if)# switchport mode trunk

ALS2# show etherchannel summary

ALS1# show interfaces trunk

ALS1# show spanning-tree

Step 4: Configure an 802.3ad LACP EtherChannel.

ALS1(config)# interface range fastEthernet 0/7 - 8

ALS1(config-if-range)# channel-group 2 mode active

Creating a port-channel interface Port-channel 2

ALS1(config-if-range)# interface port-channel 2

ALS1(config-if)# switchport mode trunk

ALS1# show etherchannel summary

Step 5: Configure a Layer 3 EtherChannel.

DLS1(config)# interface range fastEthernet 0/11 - 12

DLS1(config-if-range)# no switchport

DLS1(config-if-range)# channel-group 3 mode desirable

Creating a port-channel interface Port-channel 3

DLS1(config-if-range)# interface port-channel 3

DLS1(config-if)# no switchport

DLS1(config-if)# ip address 10.0.0.1 255.255.255.0

DLS2(config)# interface range fastEthernet 0/11 - 12

DLS2(config-if-range)# no switchport

DLS2(config-if-range)# channel-group 3 mode desirable

Creating a port-channel interface Port-channel 3

DLS2(config-if-range)# interface port-channel 3

DLS2(config-if)# no switchport

DLS2(config-if)# ip address 10.0.0.2 255.255.255.0

DLS1# ping 10.0.0.2

DLS1# show etherchannel summary

Step 6: Configure load balancing.

DLS1# show etherchannel load-balance

EtherChannel Load-Balancing Operational State (src-mac):

Non-IP: Source MAC address

IPv4: Source MAC address

IPv6: Source IP address

ALS1(config)# port-channel load-balance src-dst-mac

ALS1# show etherchannel load-balance

EtherChannel Load-Balancing Operational State (src-dst-mac):

Non-IP: Source XOR Destination MAC address

IPv4: Source XOR Destination MAC address

IPv6: Source XOR Destination IP address