P 8 Inter-VLAN Routing with an External Router

Background

Inter-VLAN routing using an external router can be a cost-effective solution when it is necessary to segment a

network into multiple broadcast domains. In this lab, you split an existing network into two separate VLANs on

the access layer switches, and use an external router to route between the VLANs.

Required Resources

• 2 routers (Cisco 1841 with Cisco IOS Release 12.4(24)T1 Advanced IP Services or comparable)

• 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or comparable)

Step 3: Configure the routers.

Router(config)# hostname ISP

ISP(config)# interface Loopback0

ISP(config-if)# ip address 200.200.200.1 255.255.255.0

ISP(config-if)# interface Serial0/0/0

ISP(config-if)# ip address 192.168.1.2 255.255.255.0

ISP(configif)# no shutdown

ISP(config-if)# exit

ISP(config)# ip route 172.16.0.0 255.255.0.0 192.168.1.1

Router(config)# hostname Gateway

Gateway(config)# interface Serial0/0/0

Gateway(config-if)# ip address 192.168.1.1 255.255.255.0

Gateway(config-if)# clockrate 64000

Gateway(config-if)# no shutdown

Gateway(config-if)# exit

Gateway(config)# ip route 0.0.0.0 0.0.0.0 192.168.1.2

Step 4: Configure the switches.

Switch(config)# hostname ALS1

ALS1(config)# interface vlan 1

ALS1(config-if)# ip address 172.16.1.101 255.255.255.0

ALS1(config-if)# no shutdown

ALS1(config-if)# exit

ALS1(config)# ip default-gateway 172.16.1.1

Switch(config)# hostname ALS2

ALS2(config)# interface vlan 1

ALS2(config-if)# ip address 172.16.1.102 255.255.255.0

ALS2(config-if)# no shutdown

ALS2(config-if)# exit

ALS2(config)# ip default-gateway 172.16.1.1

ALS1(config)# enable secret cisco

ALS1(config)# line vty 0 15

ALS1(config-line)# password cisco

ALS1(config-line)# login

ALS1(config-line)# end

ALS2(config)# enable secret cisco

ALS2(config)# line vty 0 15

ALS2(config-line)# password cisco

ALS2(config-line)# login

ALS2(config-line)# end

Step 5: Confirm the VLANs.

ALS1# show vlan

Step 6: Configure trunk links and EtherChannel on switches.

ALS1# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

ALS1(config)# interface range fastEthernet 0/11 - 12

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# channel-group 1 mode desirable

ALS1(config-if-range)# end

ALS2# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

ALS2(config)# interface range fastEthernet 0/11 - 12

ALS2(config-if-range)# switchport mode trunk

ALS2(config-if-range)# channel-group 1 mode desirable

ALS2(config-if-range)# end

ALS1# show etherchannel 1 summary

Step 7: Configure VTP.

ALS2(config)# vtp mode client

Setting device to VTP CLIENT mode.

ALS1(config)# vtp domain SWLAB

Changing VTP domain name from NULL to SWLAB

%SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to SWLAB.

ALS1(config)# vtp version 2

ALS1# show vtp status

ALS2# show vtp status

ALS1(config)# vlan 100

ALS1(config-vlan)# name Payroll

ALS1(config-vlan)# vlan 200

ALS1(config-vlan)# name Engineering

ALS2# show vlan brief

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# switchport mode access

ALS1(config-if)# switchport access vlan 100

ALS1(config-if)# spanning-tree portfast

ALS2(config)# interface fastEthernet 0/6

ALS2(config-if)# switchport mode access

ALS2(config-if)# switchport access vlan 200

ALS2(config-if)# spanning-tree portfast

ALS1# show vlan brief

ALS2# show vlan brief

ALS1(config)# interface fastEthernet 0/1

ALS1(config-if)# switchport mode trunk

ALS1(config-if)# end

Step 10: Configure the Gateway router Fast Ethernet interface for VLAN trunking.

Gateway(config)# interface fastEthernet 0/0

Gateway(config-if)# no shut

The following is a sample configuration for the VLAN 1 subinterface.

Gateway(config)# interface fastEthernet 0/0.1

Gateway(config-subif)# description Management VLAN 1

Gateway(config-subif)# encapsulation dot1q 1 native

Gateway(config-subif)# ip address 172.16.1.1 255.255.255.0

Gateway(config-subif)# interface fastEthernet 0/0.100

Gateway(config-subif)# description Payroll VLAN 100

Gateway(config-subif)# encapsulation dot1q 100

Gateway(config-subif)# ip address 172.16.100.1 255.255.255.0

Gateway(config-subif)# interface fastEthernet 0/0.200

Gateway(config-subif)# description Engineering VLAN 200

Gateway(config-subif)# encapsulation dot1q 200

Gateway(config-subif)# ip address 172.16.200.1 255.255.255.0

Gateway(configsubif)# end

Gateway# show ip interface brief

Gateway# show interfaces description

Gateway# show vlans