P 9 Modifying Default Spanning Tree Behavior

Background

Four switches have just been installed. The distribution layer switches are Catalyst 3560s, and the access

layer switches are Catalyst 2960s. There are redundant uplinks between the access layer and distribution

layer. Because of the possibility of bridging loops, spanning tree logically removes any redundant links.

Required Resources

• 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or

comparable)

• 2 switches (Cisco 3560 with the Cisco IOS Release 12.2(46)SE C3560-ADVIPSERVICESK9-M

image or comparable)

• 1 PC (optional) attached to switch ALS1.

Step 1: Prepare the switches for the lab.

DLS1(config)# interface range fastEthernet 0/7 - 12

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

Step 2: Display default spanning tree information for all switches.

DLS1# show spanning-tree

DLS2# show spanning-tree

ALS1# show spanning-tree

ALS2# show spanning-tree

Switch# show spanning-tree

Switch# conf t

Switch(config)# interface range fastEthernet 0/1-24

Switch(config-if-range)# no shutdown

Switch(config-if-range)# end

Switch# show spanning-tree

DSL1# show interfaces trunk

Step 3: Configure specific switches to be primary and secondary root.

DLS2# debug spanning-tree events

Spanning Tree event debugging is on

DLS1(config)# spanning-tree vlan 1 root primary

ALS1(config)# spanning-tree vlan 1 root secondary

DLS1# show run | include span spanning-tree mode pvst

ALS1# show run | include span

DLS1# show spanning-tree

Step 4: Change the root port using the spanning-tree port-priority command.

DLS2# show spanning-tree

DLS1# show spanning-tree

DLS1(config)# int fastEthernet 0/12

DLS1(config-if)# spanning-tree port-priority 112

DLS2# show spanning-tree

DLS1# show spanning-tree

Step 5: Configure PortFast on an access port.

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# shutdown

ALS1# debug spanning-tree events

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# switchport mode access

ALS1(config-if)# no shut

ALS1(config-if)# end

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# shutdown

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# spanning-tree portfast

ALS1(config-if)# no shutdown

ALS1(config-if)# end

ALS1# undebug all

Step 6: Change root port using the spanning-tree cost command.

ALS2# show spanning-tree

ALS2(config)# interface fastEthernet 0/10

ALS2(config-if-range)# spanning-tree cost 10

ALS2# show spanning-tree