1️⃣ Practical 1 — Single Area OSPF

Configuration

R1(config)#router ospf 1
R1(config-router)#network 10.1.100.0 0.0.0.255 area 0
R1(config-router)#network 10.1.200.0 0.0.0.255 area 0
R1(config-router)#network 10.1.1.0 0.0.0.255 area 0

Verify CLI Output

show ip protocols
show ip ospf
show ip ospf neighbor
show ip ospf interface brief
show ip route
show ip ospf database

2️⃣ Practical 2 — Multi Area OSPF

Configuration

R1(config)#router ospf 1
network 10.1.12.0 0.0.0.255 area 0

R2(config)#router ospf 1
network 10.1.12.0 0.0.0.255 area 0
network 10.1.23.0 0.0.0.255 area 23

R3(config)#router ospf 1
network 10.1.23.0 0.0.0.255 area 23

Stub Area

R2(config-router)#area 23 stub
R3(config-router)#area 23 stub

Authentication

interface serial0/0/1
ip ospf authentication
ip ospf authentication-key cisco

Verify

show ip ospf neighbor
show ip ospf database
show ip route

3️⃣ Practical 3 — RIP to OSPF Redistribution

RIP Configuration

router rip
version 2
no auto-summary
network 172.16.0.0

OSPF Configuration

router ospf 1
network 172.16.23.0 0.0.0.255 area 0

Redistribution

router rip
redistribute ospf 1 metric 4

router ospf 1
redistribute rip subnets

Verify

show ip route
show ip route rip
show ip route ospf
show ip protocols
show ip rip database

4️⃣ Practical 4 — Administrative Distance

Change Distance

router rip
distance 100

Change based on source

router ospf 1
distance 85 192.168.100.0 0.0.3.255

Verify

show ip route
show ip protocols

5️⃣ Practical 5 — BGP

ISP Router

router bgp 200
neighbor 10.0.0.2 remote-as 100
network 10.1.1.0 mask 255.255.255.0

ITA Router

router bgp 100
neighbor 10.0.0.1 remote-as 200
neighbor 172.16.0.1 remote-as 300
network 192.168.0.0
network 192.168.1.0

Verify

show ip bgp
show ip bgp neighbors
show ip route

6️⃣ Practical 6 — OSPF IPv6

Enable IPv6

ipv6 unicast-routing
ipv6 cef

OSPFv3

interface serial0/0/0
ipv6 ospf 1 area 0

Verify

show ipv6 ospf neighbor
show ipv6 route
show ipv6 interface brief

7️⃣ Practical 7 — EtherChannel

PAgP

interface range fa0/11 - 12
channel-group 1 mode desirable

LACP

interface range fa0/7 - 8
channel-group 2 mode active

Verify

show etherchannel summary
show interfaces trunk
show spanning-tree

8️⃣ Practical 8 — Inter-VLAN Routing

Router

interface fa0/0.100
encapsulation dot1q 100
ip address 172.16.100.1 255.255.255.0

interface fa0/0.200
encapsulation dot1q 200
ip address 172.16.200.1 255.255.255.0

Switch

vlan 100
name Payroll

vlan 200
name Engineering

Verify

show vlan brief
show ip interface brief
show interfaces trunk

9️⃣ Practical 9 — Spanning Tree

Primary Root

spanning-tree vlan 1 root primary

Secondary Root

spanning-tree vlan 1 root secondary

Portfast

interface fa0/6
spanning-tree portfast

Verify

show spanning-tree
show interfaces trunk

⭐ MOST IMPORTANT CLI COMMANDS FOR EXAM

Always run these to show output screenshots:

show ip route
show ip protocols
show ip ospf neighbor
show ip ospf interface brief
show ip bgp
show ipv6 route
show etherchannel summary
show vlan brief
show spanning-tree